Compressor Station Startup Procedure

Standard Operating Procedure | Capstone Compressor Station Model

| Document No. | VR-OPS-001 | Revision | 3 |
| --- | --- | --- | --- |
| System | Compression Station Training Model | Effective Date | 25-Mar-2026 |
| Prepared For | Capstone Design | Status | Uncontrolled Academic Document |

|  | Use Limitation This document is formatted to reflect professional operating-document practice for academic simulation use. It is not intended for live plant, commissioning, or field execution. |
| --- | --- |

| 1. Purpose Define a clear, controlled, and repeatable startup sequence for the compressor station model, including operator actions, verification points, and expected system responses. | 2. Scope Applies to the capstone compressor station simulation, including scrubbers, feed control, SPV, ASV-1, ASV-2, NRV-1, NRV-2, Compressor 1, Compressor 2, Intercooler 1, and Intercooler 2. |
| --- | --- |

3. Required PPE and Operator Readiness

Minimum PPE icons are shown below to reflect a realistic compressor-station operating procedure format.

| Hard Hat | Safety Glasses | Hearing Protection | Work Gloves | Safety Boots |
| --- | --- | --- | --- | --- |

| 4. Preconditions • Simulation is loaded and ready for operation. • All controls, dials, and status indicators are available. • Compressors are in standby status and intercoolers are initially off. • Scrubber level indication is visible to the operator. • No abnormal alarm, trip or fault indication is present. | 5. General Operating Notes • Execute the startup sequence in the order specified. • Perform gradual valve movements smoothly where noted. • Confirm expected response after each major action before proceeding. • Pause the sequence using ESD red button if abnormal noise, inconsistent status, or unexpected response is observed. |
| --- | --- |

6. Startup Procedure

| 6.1 Drain Scrubbers |  |
| --- | --- |
| Action • Drain each scrubber individually using its dedicated drain controls • Open BD-1 to 100% by turning the BD-1 dial to maximum. • Open BD-2 to 100% by turning the BD-2 dial to maximum. • Observe the scrubber level while each drain action is in progress. | Expected confirmation • Scrubber level decreases visibly during each drain action. • Each scrubber responds independently to its respective control. |
| Purpose: reduce liquid inventory before advancing the startup sequence. |  |

| 6.2 Open Recycle Valves |  |
| --- | --- |
| Action • Open ASV-1 to 100% by turning the ASV-1 dial to maximum. • Open ASV-2 to 100% by turning the ASV-2 dial to maximum. | Expected confirmation • ASV-1 indicates fully open. • ASV-2 indicates fully open. • Recycle flow path is established in the startup configuration. |
| Maintain full recycle during early startup to reflect a conservative compressor loading condition in the simulation. |  |

| 6.3 Open Feed |  |
| --- | --- |
| Action • Open feed to 100% by turning the feed dial to maximum. | Expected confirmation • Feed path is fully established. • System is aligned for initial compressor startup. |

| 6.4 Set SPV and Start Compressor 1 |  |
| --- | --- |
| Action • Set SPV to 10-25% open. • Press the green start button for Compressor 1. • Turn the Compressor 1 dial to maximum. | Expected confirmation • Compressor 1 changes from standby to running. • Compressor noise becomes audible and increases as the compressor dial is raised. • No abnormal trip or fault indication appears. |

| 6.5 Increase SPV |  |
| --- | --- |
| Action • Open SPV to 40-50% by turning the SPV dial to the intermediate position. | Expected confirmation • SPV reaches the intermediate position. • Compressor 1 remains stable as system flow increases. |

| 6.6 Reduce ASV-1 |  |
| --- | --- |
| Action • Slowly close ASV-1 from 100% to 50%. • Make the adjustment smoothly over at least 4 seconds. | Expected confirmation • ASV-1 transitions gradually to 50%. • System response remains stable during recycle reduction. |
| Caution: this change shall not be performed abruptly. |  |

| 6.7 Open NRV-1, Start Compressor 2, and Start Intercooler 1 |  |
| --- | --- |
| Action • Open NRV-1 to 40-50%. • Press the green start button for Compressor 2. • Turn the Compressor 2 dial to maximum. • Turn the Intercooler 1 dial to 60%. | Expected confirmation • NRV-1 reaches the required intermediate position. • Compressor 2 changes from standby to running and compressor noise increases as the dial is raised. • Intercooler 1 lights turn on and flash red and blue. • Intercooler 1 changes from off to active status. |
| Verify Compressor 2 and Intercooler 1 status before advancing to the next step. |  |

| 6.8 Reduce ASV-2 |  |
| --- | --- |
| Action • Slowly close ASV-2 from 100% to 50%. • Make the adjustment smoothly over at least 4 seconds. | Expected confirmation • ASV-2 transitions gradually to 50%. • System remains stable during the recycle reduction. |
| Caution: do not make a rapid reduction. |  |

| 6.9 Open NRV-2 and Start Intercooler 2 |  |
| --- | --- |
| Action • Open NRV-2 to an initial opening of approximately 5-10%. • Turn the Intercooler 2 dial to 60%. | Expected confirmation • NRV-2 begins opening gradually. • Intercooler 2 lights turn on and flash red and blue. • Intercooler 2 changes from off to active status. |
| This step introduces the second downstream path gradually before final opening. |  |

| 6.10 Open NRV-1 Fully |  |
| --- | --- |
| Action • Turn NRV-1 to 100%. | Expected confirmation • NRV-1 indicates fully open. • The first-stage downstream flow path is fully established in the simulation. |

| 6.11 Open SPV Fully and Further Reduce ASV-1 |  |
| --- | --- |
| Action • Open SPV to 100%. • Close ASV-1 to 5%. | Expected confirmation • SPV indicates fully open. • ASV-1 reaches 5%. • Compressor response remains steady during the transition. |
| Observe compressor stability closely while reducing recycle to the near-minimum startup position. |  |

| 6.12 Open NRV-2 Fully and Further Reduce ASV-2 |  |
| --- | --- |
| Action • Slowly turn NRV-2 to 100%. • Close ASV-2 to 15%. | Expected confirmation • NRV-2 indicates fully open. • ASV-2 reaches 15%. • Both compression stages and both intercoolers are in service. |
| This completes the staged transition from full recycle startup toward the intended final operating configuration. |  |

7. Final Operating Confirmations

| Startup completion criteria • Scrubber levels were reduced during the drain step. • Feed is at 100%. • ASV-1 is at 5% and ASV-2 is at 15%. • SPV is at 100%. • NRV-1 is at 100% and NRV-2 is at 100%. • Compressor 1 and Compressor 2 are running. • Intercooler 1 and Intercooler 2 are active. • Intercooler status lights are flashing red and blue as intended. • No abnormal trips, faults, or inconsistent indications are present in the simulation. | Acceptance Startup sequence complete Both compressors online Both intercoolers online System stable |
| --- | --- |

10. Revision History

| Rev | Date | Prepared By | Description |
| --- | --- | --- | --- |
| 0 | 18-Mar-2026 | Ahmed Elkhrashy | Initial issue for simulation and presentation use |
| 1 | 19-Mar-2026 | Zain Tareq | Added Purpose and Scope |
| 2 | 21-Mar-2026 | Zaid Ahmed | Added Header with the VR OPS Logo |
| 3 | 25-Mar-2026 | Ahmed Elkhrashy | Clarified that the document is limited for simulation use and not applicable for live use. |